## Introduction

Micro:bit gamepad is an expansion gamepad based on micro:bit. You only need to plug in the micro:bit, it will turn into a wireless remote controller or a wireless game console. This product packaged with acrylic plate, that giving it a good feel and no longer feels like a bare circuit board. The gamepad has a total of 8 buttons, the left side have up, down, left, right four buttons, the right side has X, Y two buttons, and two buttons A, B are in the front of the gamepad. The gamepad also has programmable built-in vibration motor, buzzer, and LED. Using graphical programming, it will turn into a multimedia vibration controller or multimedia interactive game console instantly. The programming platform supports MakeCode graphical programming and python. It is a very suitable tool for both beginners and masters.

## Specification

- Operating Voltage: 3VDC (2 AAA batteries)

- Number of Buttons: Up, Down, Left, Right, X, Y, A, B Total 8 programmable buttons

- Onboard Vibration motor × 1

- Onboard LED × 1

- Onboard Buzzer × 1

- Acrylic Floor x 1

- Acrylic Panel × 1

- Battery Box with Cover × 1

- Velcro × 2

- Copper Cylinder × 4

- Screw × 8

- Big Button ×6

- Small Button× 2

- Dimension:148x57(mm)/ 5.83*2.24in

## Board Overview

| Function | Description |
| --- | --- |
| LED | Control pin P16 |
| Vibration Motor | Control pin P12 |
| Buzzer | Control pin P0 |
| PWR | Power Indicator |
| Switch | Turn off the power |
| Opearting Voltage | 3V~3.7V（2x AAA batteries） |

## Tutorial

## MakeCode Graphical Programming

The basic tutorial of MakeCode:

- The library of gamepad: https://github.com/DFRobot/pxt-gamePad

- Enter MakeCode

- Find Advanced > Add package

- Copy the library link and paste it in the option box

- Click the gamepad, and you can find a new icon on the right.

## The programming of event trigger

Result: When the X is pressed, the vibration motor vibrates and the LED lights up; when the X is released, the vibration motor stops and the LED turns off.

Result: When the X is pressed, the vibrating motor vibrates and the LED lights up; when the Y is pressed, the vibrating motor stops and the LED turns off.

### The Programming of Cyclic Query

Result: When the X is pressed, the vibration motor vibrates at the strength of 175, and when the X is released, the vibration motor stops vibrating.

### Rock-Paper-Scissors

Graphical source program: Rock-Paper-Scissors Execution of the Program:

- First, you should set the two main boards are in the 1th wireless group, so that they can communicate with each other.

- When the left button is pressed, LED dot matrix displays the scissors, and sends the number 1.

- When the down button is pressed, LED dot matrix displays the rock, and sends the number 2.

- When the right button is pressed, LED dot matrix displays the paper, and sends the number 3.

- In the infinite loop, compare the wireless received numbers with your own Numbers;if you lost,it will display “×", if you win, it will show "√".

- After showing the result, clear all data.

### Yacht

Graphical source program: Dice Game  Execution of the Program:

- Set the wireless to group 1 when booting, and initialize item to 0.

- When the vibration is detected, turn off the LED and initialize the value to 0, and use the acceleration value to produce a random number between 1 and 6.

- Incremental display from 0 to randomly generated number on the LED dot matrix, play the pitch and vibrate motor.

- Send 1~6 random numbers to other main boards via wireless.

- Comparing the random number with the received number. If it is less than the received number, turn off the light, otherwise turn on the light.

### Remote Control Handle

Graphical source program of handle: Gamepad Graphical source program of car: Mobile Platform

Execution of the Program:

- Set the wireless to group 1.

- When different buttons are pressed, send different numbers to let the car performs different actions.

- When the button is released, the number 0 is sent to make the car stop.

Execution of the Program:

- Set the wireless to group 1.

- When receiving different numbers, let the car perform different actions such as forward, backward, turn left, turn right