Building a Piano with Micro:Bit

The Hook-Up

Theory

Pushbutton switches are connect as shown.  One side of the switch is connected to 3.3 V on the Micro:Bit and the other side of the switch is connected to ground via a 10K Ω resistor.  When the switch is open, the GPIO (General Purpose Input Output) is at 0 V (logic 0).  When the switch is pushed, the GPIO pin is at 3.3 V (logic 1).  The Micro:Bit has 17 GPIO Pins.  They are pin 0 to 16.  Many of these pins are shared with other devices however a few are dedicated GPIO (pins 8 and 12).  Pin 5 is shared with button A and is connected to 3.3 V so detect an external button push, it must be pulled down to 0 V

The Code